LAB 2A in Mac OS

- Pre-Lab

- In Mac OS, We first touch a hello.txt and then edit the hello.txt by vim.At last, we can see the document through cat.

- Setting Lab 2 A

- First, Install Homebrew

- /bin/bash -c "$(curl -fsSL https://raw.githubusercontent.com/Homebrew/install/HEAD/install.sh)"

- Second, Export Homebrew to environment path:

- cd ~(open ~directory)

- If we don’t have .zshrc document, we need to creat in ~~directory by touch .zshrc

- open or vim .zshrc to edit the file

- add two exports to the file

- export PATH=/opt/homebrew/bin:$PATH

- export PATH=/opt/homebrew/sbin:$PATH

- source .zhsrc

- brew -v to check.If the HomeBrew is successfully export to environment, we will look the version of HomeBrew.

- Third, From the above figure, we need to install the wget by

- brew install wget

- Fourth, install the toolchain:

brew install cmake

brew tap ArmMbed/homebrew-formulae

brew install arm-none-eabi-gcc

- Then, set environment in VsCode

- By following the guidence in document Getting start with Raspberry 9.1:

- (1)After starting Visual Studio Code you then need to install the CMake Tools extension. Click on the Extensions icon in the left-hand toolbar (or type Cmd + Shift + X), and search for "CMake Tools" and click on the entry in the list, and then click

on the install button.

- We now need to set the PICO_SDK_PATH environment variable. Navigate to the pico-examples directory and create a .vscode directory and add a file called settings.json to tell CMake Tools to location of the SDK. Additionally point Visual Studio at the CMake Tools extension.

{   "cmake.environment":

{   "PICO_SDK_PATH":"../../pico-sdk"   },

}

- Then go to the File menu and click on "Add Folder to Workspace…" and navigate to pico-examples repo and click "Okay". The project will load and you’ll (probably) be prompted to choose a compiler, Select "GCC for arm-noneeabi"

- clicke the bulid botton  we can see:

- In the end, we need to Drag and drop hello_usb.uf2 onto the Mass Storage Device from the

- pico-examples/build/hello_world/usb/

- Then, open the terminal and link to the screen .

- we can see that